VOLUME 4.2

FINANCIAL OFFER TEMPLATES

LUMP SUM CONTRACTS

Introduction

1.	The breakdown of the lump-sum price (Volume 4.2.3) is the itemised list of prices showing the build-up of the price in a lump-sum contract. This breakdown of the lump-sum price does not derogate in any way to the clause stating that, in a lump-sum contract, the total contract price remains fixed irrespective of the quantity of work actually carried out.

The amounts due will be calculated:

The breakdown must coincide with the payment-definition chosen in Article 49 of the special conditions: by the tranches specified in Article 49(1)(a) of the special conditions.

2.	The item description given in the breakdown of the lump-sum price in no way limits the contractor’s obligations under the contract to provide all the works described elsewhere.

3.	The prices of the breakdown of the lump-sum price include all incidental and contingent expenses and all risks necessary to construct, complete and maintain all works in accordance with the contract. Unless separate items are provided in the breakdown of the lump-sum price, prices include all costs involved in the various items of the breakdown.

4.	The lump–sum price and the prices of the breakdown of the lump-sum price are all-inclusive and include any non-exonerated tax or fiscal duty.

VOLUME 4.2.3 — BREAKDOWN OF THE LUMP-SUM PRICE

Establishing a multipurpose recreation playground around Lake Gradche

NB! While preparing the financial offer, tenderers should consider the obligation of preparing and installing a construction info board according to the National legislation. Additionally, the tenderer should plan to purchase and instal a durable plaque clearly visible to the public, presenting the emblem of the European Union in accordance with the technical characteristics laid down in Annex IX of Regulation (EU) 2021/1060. The design of the construction info board and the plaque must be previously approved by the Contracting Authority

| Item | DECSRIPTION | Unit | Unit price EUR | Firm quantity | Total EUR |
| --- | --- | --- | --- | --- | --- |
| 1. EARTHWORKS |  |  |  |  |  |
| 2 | Planning and clearing the terrain and marking the building with staking P=1296.75 | m² |  | 1.296,75 |  |
| 3 | Mechanical wide earth excavation 1296.7x0.44 loading and transporting the material to the nearest landfill max-10km | m³ |  | 570,60 |  |
| 4 | Machine compaction of the subsoil by rolling only on the playing field | m² |  | 1.083,75 |  |
| 5 | Machine narrow earth excavation for foundation strips and drainage ditch 70% loading and transport of the material to the nearest landfill max-10km | m³ |  | 192,20 |  |
| 6 | Manual digging and leveling of drainage trench 30% and loading and transport to landfill | m³ |  | 108,50 |  |
| 7 | Manual excavation for single foundations for reflector poles loading and transport of the material to the nearest landfill max-10km | m³ |  | 8,73 |  |
| 8 | Manual excavation for an access path from behaton loading and transport to the nearest landfill max-10km | m3 |  | 1,40 |  |
| 9 | Earth filling after concreting the foundations and around the building, with earth filling in layers of 25-30 cm with compactors to the required compaction of module 60 Mpa around foundation strips | m³ |  | 16,94 |  |
| 10 | Procurement, transport and installation of polypropylene geotextile of 200g/m2 before filling gravel under the entire terrain | m² |  | 1.020,66 |  |
| 11 | Procurement, transportation, and installation of 125g/m² polypropylene geotextile prior to backfilling in a drainage trench | m² |  | 494,83 |  |
| 12 | Procurement, placing, and compacting of gravel in layers, with compaction to achieve a required modulus of 60 MPa beneath foundation strips measuring 71.91 x 0.2 meters | m³ |  | 14,38 |  |
| 13 | Procurement, piling and rolling of gravel in layers, with compaction to the required compaction modulus of 60MPa under single foundations 0.3x1.2x4 | m³ |  | 1,73 |  |
| 14 | Procurement, placement, and compaction of subbase material with crushed stone 0-63 under asphalt pavement, compacted to achieve the required density of 70 MPa, thickness of 35 cm, on a slope of 0.5%-1%, area 1020.66 m² | m³ |  | 357,23 |  |
| 15 | Procurement, transportation, and spreading of a layer of sand at the bottom of the drainage trench with a thickness of 10 cm, area 142x1.4x0.1 m. | m³ |  | 19,88 |  |
| 16 | Procurement, placement, and compaction of gravel in layers for a drainage trench with gravel size 8-12 mm, thickness 50 cm, on a slope of 0.5%-1%. | m³ |  | 90,30 |  |
| 17 | Procurement and layering of sand in a drainage trench with gravel size 4-8 mm, thickness of 50 cm, on a slope of 0.5%-1%. | m³ |  | 90,30 |  |
| 18 | Procurement, placement, and compaction of sand, particle size 0-4 mm, as a base for a perimeter path paved with behaton, following prior placement of gravel drainage, particle size 0-4 mm, thickness 40 cm | m³ |  | 79,50 |  |
| 19 | Procurement, placement, and compaction of sand, particle size 0-4 mm, as a base for an access pathway paved with behaton, following prior compaction of soil measuring 0.08 x 2.5 x 4.00 meters. | m³ |  | 0,80 |  |
| 20 | Sterilization of the field with means to prevent the growth of weeds and the development of insects | m² |  | 1.296,75 |  |
|  | TOTAL POS.1 |  |  |  |  |
| 2. CONCRETE AND REINFORCEMENT |  |  |  |  |  |
| 1 | Procurement, transport and installation of concrete in structures with necessary formwork and support |  |  |  |  |
| 1.1 | Reinforced foundations, MB 30 in required formwork, strip foundations 71.90x0.85 | m³ |  | 61,12 |  |
| 1.2 | Individual foundations for lighting reflector poles, 1.20 x 1.20 x 1.40 meters each, quantity 4. | m³ |  | 8,06 |  |
| 1.3 | Individual foundations for lighting reflector poles, 1.20 x 1.20 x 1.40 meters each, quantity 4. | m³ |  | 1,80 |  |
| 2 | Lean concrete MB10 |  |  |  |  |
| 2.1 | Individual foundations for floodlight poles 1.20x1.20x0.10 x 4 | m³ |  | 0,57 |  |
| 2.2 | Foundation strips 71,90x0,10 | m³ |  | 7,19 |  |
| 2.3 | Individual foundation for tennis net installation 0,37x4x0,10 | m³ |  | 0,15 |  |
| 3 | Procurement, cutting, bending and installation of concrete iron RA 400/500 | kg |  | 1.530,00 |  |
| 4 | Anchors M20 | piece |  | 32,00 |  |
| 5 | Anchors M20 | piece |  | 32,00 |  |
|  | Total POS 2 |  |  |  |  |
| 3. ASPHALT WORKS |  |  |  |  |  |
| 1 | Asphalting of base layer d=5cm BNS 22 / slope min 0.5% max 1% | m² |  | 1.083,75 |  |
| 2 | Asphalting of base layer d=3cm / AB 8mm / slope min 0.5% max 1% | m² |  | 1.083,75 |  |
|  | Total POS. 3 |  |  |  |  |

| 4. METAL - LOCK WORKS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Procurement, transport, manufacturing and installation of the fence around the fields according to the project, by: |  |  |  |  |
| 1 | Filled with universal wire steel plasticized mesh - approx 50/50mm Ф=2.0mm with dark green color fixed (maximum tension) with metal galvanized clamps (taken area without waste) | m² |  | 545,56 |  |
| 2 | Double-wing entrance door 215/190 made of Fe profile 50/50 with filling of universal wire steel plasticized mesh oksa 50/50mm Ф=2.0mm with locking lock | piece |  | 1,00 |  |
| 3 | 1. Procurement, transport, manufacturing and installation of: Fence made of iron box profiles 100/80/4 anchored in foundation strips H=400 and 500 cm. Minimized and grease-painted in two layers in dark gray color, fence posts Fe box profile 100/80/4 mined and grease-painted in two layers in dark gray color (see construction details) 2. Posts for reflectors iron profiles f219 with narrowing from f101,6 and H=12m with anchor tiles (according to detail) quantity - 4 are lead coated and grease painted in two layers in dark gray color 3. Posts for baskets Fe f160 with plate 400.400.10 per detail quantity - 4 are lead coated and painted in two layers in dark gray color | kg |  | 9.770,00 |  |
|  | Total POS.4 |  |  |  |  |
| 5. EQUIPMENT FOR MULTIFUNCTIONAL PLAYGROUND |  |  |  |  |  |
| 1 | Procurement, transport and installation of metal benches made of perforated steel sheet with two seats and a backrest. In dark green color Fig.1 | piece |  | 4,00 |  |
| 2 | Procurement and installation of a trash can (fixed on a pole) Fig. 2 | piece |  | 2,00 |  |
| 3 | Procurement and installation of a free-standing goal with a goal net with all binding elements and dimensions according to IHF standards goal bar Fig.3 | piece |  | 2,00 |  |
| 4 | Procurement and installation of a fiberglass basketball board with hoop and net and all binding elements according to FIBA ​​standards Fig.4 | piece |  | 4,00 |  |
| 5 | Procurement and installation of polyethylene wind canvases with printed inscriptions on one side in 4 colors up to 2m high | m² |  | 135,00 |  |
| 6 | Purchase of handball balls that meet IHF ​​standards | piece |  | 4,00 |  |
| 7 | Purchase of basketball balls that meet FIBA ​​standards | piece |  | 4,00 |  |
| 8 | Purchase of soccer balls that meet FIFA ​​standards | piece |  | 4,00 |  |
| 9 | Waste Bin Supply of materials, manufacturing, transportation, and installation or supply, transportation, and installation of a waste bin with dimensions 50/50/82 cm, made of a steel structure with a profile of 40/40/3 mm, a box made of tin with a thickness of 0.8 mm, which is covered with treated wooden planks with a profile of 120/21 mm. The structure must be anchored to a concrete foundation MB30 with dimensions 30/30/40 cm. The waste container should be made of galvanized tin with a thickness of 0.5 mm. The bin must have a finished lid made of tin with a thickness of 3 mm, with dimensions 500/500 mm and an opening of Ф300 mm. The structure should be lead painted and painted with graphite paint. (according to the drawing) The color of the wooden part and the structure is defined by the investor. | piece |  | 3,00 |  |
| 10 | Gazebo Construction of a hexagonal canopy, with the height of the lowest part at 2.4 m and at the center 3 m. Made of metal profiles—6 columns of 8x8 cm with a mutual distance of 218 cm, placed on an AB slab, thickness 15 cm, MB30, and a roof structure in the shape of a hexagon with sides of 296 cm made of profiles 5x8 cm and smaller profiles for supporting OSB boards, covered with OSB boards of 11 mm thickness and topped with Canadian shingles (dark green). The profiles are first protected with a corrosion-resistant agent and then painted with anthracite. A metal seating structure should be made along the entire length at the base, allowing for entry, meaning the hexagon should serve as a seating area and have horizontal pipes for support. For seating, there should be 4 treated planks of 8x5 cm, and on the back 2 of 12x5 cm. All painted with Sandolin and lacquered in two coats. In the middle, a table in the shape of a hexagon should be constructed with a height of 80 cm, as well as a wooden tabletop, sized according to the drawing. | piece |  | 2,00 |  |
| 11 | Park Benches with Tables Supply of materials, manufacturing, transportation, and installation of park benches with tables, with dimensions and shape defined according to the technical drawing (attached). The supporting structure should be made of metal with a profile of 4x4 cm, coated with a corrosion-resistant agent and painted in a color of the investor's choice, and it should be anchored to a concrete foundation MB30 with dimensions 30/30/40 cm. Wooden planks of 5x8 cm for seating and a tabletop made of sawn timber should be placed on the structure, crafted through cutting, milling, planing, screwing, staining, and varnishing with boat varnish in a minimum of 2 coats | piece |  | 9,00 |  |
|  | Total POS. 5 |  |  |  |  |
| 6. OTHER WORKS |  |  |  |  |  |
| 1 | Construction of a roundabout and access path from 10/20/6cm bechaton elements with final edge elements by pouring the asphalt joint with concrete | m² |  | 213,00 |  |
| 2 | Construction of drainage with prefabricated small channels for surface drainage of atmospheric water smoke 40/40/10cm | m |  | 134,40 |  |
| 3 | Procurement and installation of illuminated advertising boards made of plexiglass 200x80 with all necessary fastening elements and all necessary electrical elements | piece |  | 1,00 |  |
| 4 | Cleaning of the facility during construction and after completion of the works | Lump sum |  | 1,00 |  |
| 5 | Marking of the handball and basketball court with quality asphalt paint | Lump sum |  | 1,00 |  |
|  | Total POS.6 |  |  |  |  |
| 7. ELECTRICAL INSTALLATIONS |  |  |  |  |  |
| 1 | Excavation of a trench in category III and IV ground for the construction of a route in which FeZn 30x4mm will be laid. The trench should be 0.4m wide at the bottom and 0.8m deep. After the excavation is completed, fine river sand or loam soil is sprinkled in a layer of 10 cm. before laying and 10 cm after laying the cable. After laying the cable, loose earth is sprinkled in a layer of 10 cm with compaction. A GAL shield is placed on the cable and a thermoplastic warning tape "CAUTION ENERGY CABLE" is laid 40 cm from the cable and after laying the tape the trench is filled with the basic material | m |  | 154,00 |  |
| 2 | Delivery and installation of an external separate cable cabinet RO-4 made of double-decaled sheet metal, one-piece with degree of protection IP54 for installation on a concrete foundation with openings for the entry and exit of power cables. The exact dimensions will be determined after the production of the assembly drawings, taking into account that 10% spare dimensional space will remain. In the closet will be installed: |  |  |  |  |
|  | three-pole socket for fuses NV-0-125/ | piece |  | 7,00 |  |
|  | fuses NV 0 125/32A | piece |  | 3,00 |  |
|  | fuses NV 0 125 /25A | piece |  | 12,00 |  |
|  | fuses NV 0 125/16A | piece |  | 6,00 |  |
|  | fuses B10A-1p | piece |  | 1,00 |  |
|  | fuses B16A-1p | piece |  | 8,00 |  |
|  | fuses C6A-1p | piece |  | 1,00 |  |
|  | switch DPX 3P In=125A, It=32A | piece |  | 1,00 |  |
|  | cam clutch БС - 10 - 52 У | piece |  | 4,00 |  |
|  | cam clutch БС - 10 - 51У | piece |  | 2,00 |  |
|  | contactors CN |  |  |  |  |
|  | CN -20; 380V | piece |  | 2,00 |  |
|  | CN -25; 380V | piece |  | 4,00 |  |
|  | CN-10; 220V | piece |  | 2,00 |  |
|  | timer | piece |  | 1,00 |  |
|  | overvoltage protection | piece |  | 1,00 |  |
|  | additional equipment for reflectors pre-connected handle B1000HI/HS | piece |  | 8,00 |  |
|  | capacitors 40mF | piece |  | 16,00 |  |
|  | for everything complete with making a foundation, built-in pipes Ф 60 - 5 and installation of a cabinet is paid per piece | piece |  | 1,00 |  |
| 3 | Procurement, delivery and laying of energize cable type NAYY-4X25 mm 2+16mm2. The cable is laid in a trench in the ground (from the previous position) connecting in RO-4 and RO on a lamp post. It is paid per meter | m |  | 60,00 |  |
| 4 | Procurement, delivery and laying of energized cable type NYY-J-3X4 mm2. The cable is laid in a trench in the ground (from the previous position) by connecting RO-4 and a lamp. It is paid per meter. | m |  | 360,00 |  |
| 5 | Procurement, delivery and laying of galvanized tape FeZn30x4 mm in the same trench, parallel to the energy cable from the previous position for grounding the poles. It is paid per meter | m |  | 160,00 |  |
| 6 | Installation of rigid plastic pipe F63 with length L=6m is paid per piece | piece |  | 4,00 |  |
| 7 | Installation of anchor bolts supplied with the poles and plastic hoses F50 L=2m for cable entry and exit. Everything ready for work is paid per piece | piece |  | 4,00 |  |
| 8 | Delivery and installation of a bracket with end lengths of 3 meters, made of heavy-duty profile 60x60mm according to detail A drawing No. 5, is paid per piece. | piece |  | 4,00 |  |
| 9 | Delivery and installation of a 1000W HID reflector, complete with igniter and metal halide bulb. Payment is per unit for the entire set. The origin of the components must be EU or USA. | piece |  | 8,00 |  |
| 10 | Testing of the installation with issuance of necessary certificates and commissioning. | lump sum |  | 1,00 |  |
|  | GROUNDING OF A FENCE |  |  |  |  |
| 11 | Procurement, delivery, and installation of galvanized strip FeZn30x4 mm in a trench designated for grounding the fence. Payment is per meter. | m |  | 248,00 |  |
| 12 | Manufacturing of a joint for the foundational grounding conductor with a metal fence, executed with MKC.NB.4 932. Payment is per number of joints. | num |  | 12,00 |  |
| 13 | Examination of an installation by measuring ground resistance and issuance of a report by a licensed institution | lump sum |  | 1,00 |  |
|  | Total POS.7 |  |  |  |  |
| 8. ATMOSPHERIC SEWERAGE |  |  |  |  |  |
| 1 | Procurement, transportation and installation of drainage pipes corrugated pipes F200 with triple angular perforation angle of 120° with all necessary joint and sealing material | m |  | 136,00 |  |
| 2 | CONCRETING WORKS |  |  |  |  |
| 2,1 | Construction of inspection manholes 110/110/187 cm from compacted concrete MB30 with the manufacture of formwork plastered from the inside with cement mortar 1:2 and installation of the required number of concrete iron ladders Ф18 mm from the top to cover the manhole with a cast cover with a light opening F60 light type. Double reinforced with mesh reinforcement Q188 | piece |  | 1,00 |  |
| 2,2 | Production of revision manholes 110/110/204 cm from compacted concrete MB30 with the manufacture of formwork plastered from the inside with cement mortar 1:2 and installation of the required number of concrete iron ladders Ф18 mm from the top to cover the manhole with a cast cover with light aperture F60 light type. Double reinforced with mesh reinforcement Q188 | piece |  | 1,00 |  |
| 2,3 | Production of inspection manholes 110/110/260 cm from compacted concrete MB30 with the manufacture of formwork plastered from the inside with cement mortar 1:2 and the installation of the required number of concrete iron ladders Ф18 mm from the top to cover the manhole with a cast cover with light aperture F60 light type. Double reinforced with mesh reinforcement Q188 | piece |  | 1,00 |  |
|  | Total POS.8 |  |  |  |  |

|  | 9. PROCUREMENT OF A PEDAL BOAT |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Procurement of pedal boats for 4 persons Material: Rotomolded polyethylene Length: min. 280 cm Width: min. 150 cm Weight: max 120 kg Persons: min. 4 persons Min. Load: 500 kg Equipment: Min. 4 x seats 1 x control paddle Min. 4 x pedals One captain’s horn 2 x paddle wheels The attached image is for informational purposes only. The offered PEDAL BOAT can be in any shape and color. | num. |  | 2,00 |  |
|  | Total POS. 9 |  |  |  |  |

| RECAPITULAR |  | Amount in EUR |
| --- | --- | --- |
| 1 | Earthworks |  |
| 2 | Concrete and reinforcement |  |
| 3 | Asphalt works |  |
| 4 | Metal lock works |  |
| 5 | Equipment for multifunctional playground |  |
| 6 | Other works |  |
| 7 | Electrical installations |  |
| 8 | Atmospheric sewerage |  |
| 9 | Procurement of pedal boats |  |
|  | TOTAL |  |
|  | VAT18% |  |
|  | Total lump-sum price (with VAT) |  |

Tenderer:

___________________________